Department of Mechanical Engineering

Sub: Heat Transfer     TE B Div A.Y 2019-20

Assignment no 04

CO4: Develop mathematical model for transient heat transfer

- Explain Hydrodynamic and thermal boundary layer

- For natural convection prove that Nu = C (Gr.Pr)

- Estimate the heat transfer rate from a 100 W incandescent bulb at 140°C to an ambient at 24°C. Approximate the bulb as 60 cm diameter sphere. Calculate the percentage of power lost by natural convection. Use following correlation and air properties:

Nu = 0.60 (Gr.Pr)1/4

The properties of air at 82°C are v = 21.46 × 10-6 m2/s, K = 30.38 × 10-3 W/mK, Pr = 0.699

- Explain non-dimensional number used in convective heat transfer.