Department of Mechanical Engineering

Sub: Heat Transfer     TE B Div A.Y 2019-20

Assignment no 06

CO6: Analyse different heat exchangers and quantify their performance

- Derive the relationship between the effectiveness and the number of transfer units for a parallel flow heat exchanger.

- Hot oil is being cooled from 160ºC to 110ºC in a parallel flow heat exchanger by water entering at 24ºC and exiting at 60ºC. Determine the outlet temperature of both streams if the heat exchanger is made counter flow.

- Water (mass = 1.4 kg/s, Cp = 4.187 KJ/Kg-K) at 160 ºC is cooled by water (m = 1500 kg/hr and Cp = 4.187 KJ/ Kg) entering at 35 ºC.

Determine capacity ratio, NTU and effectiveness if the overall heat transfer coefficient is 300 W/m2K. Assume

parallel flow.